MIDAS D5L6

Linear De-Lidder System

Technical Data Sheet

The Midas D5L6 is a bench-top linear de-lidder for controlled removal of hermetic package lids in microelectronics assembly and rework. The system uses an enclosed rotary cutter with interlocked lid, motion-controlled X/Y positioning, and integrated ULPA-filtered particle extraction.

# Physical

| Parameter | Specification |
| --- | --- |
| Machine footprint (W × D × H) | 24″ × 22″ × 14″ (610 × 559 × 356 mm) |
| Recommended clear work area | 30″ × 24″ × 26″ (762 × 610 × 660 mm) |
| Machine weight | 141 lbs (64 kg) — two-person lift |
| Bench requirement | ≥ 250 lbs capacity; solid, level, non-vibrating |
| Construction | Bench-top enclosure with interlocked access lid |

# Electrical

| Parameter | Specification |
| --- | --- |
| Supply voltage | 110 V AC, 60 Hz, single phase (factory-configured) |
| Alternate configuration | 230 V AC, 50 Hz, single phase (build option) |
| Current draw | __________ A [confirm from nameplate] |
| Connection | NEMA 5-15 grounded outlet, cord supplied |
| Circuit requirement | Separate from sensitive instrumentation circuits |
| Grounding | Single chassis ground point with ESD wrist-strap connector |

# Control System

| Parameter | Specification |
| --- | --- |
| Controller | Beckhoff industrial PC (TwinCAT 3) |
| Operator interface | Integrated touchscreen HMI |
| Recipe management | Stored recipes for repeatable package setups |
| Motion | Applied Motion stepper drives and motors |

# Process Utilities

| Parameter | Specification |
| --- | --- |
| Compressed air / nitrogen / water | None required — dry process |
| Particle control | Self-contained ULPA-filtered vacuum unit (included) |
| Vacuum connection | 1.5″ ID ESD-conductive hose to machine vacuum outlet |
| Cleanroom central vacuum | Optional — 1.5″ conductive outlet in lieu of supplied unit |
| Vent / exhaust | Not required (ULPA filtration) |

# Environmental

| Parameter | Specification |
| --- | --- |
| Operating temperature | ≤ 40 °C |
| Operating humidity | ≤ 90% RH, non-condensing |
| Atmosphere | Non-explosive |
| Installation environment | In or adjacent to a microelectronics clean room |

# Safety

| Feature | Detail |
| --- | --- |
| Cutter enclosure | Fully enclosed cutter with dual lid interlocks |
| Interlock behavior | Cutter stops when lid is opened |
| Tool change | Spindle lock provided in tool kit |
| Operator PPE | Safety glasses recommended |
| ESD | Chassis ground point; ESD controls per customer microelectronics practice |

# Process Capability

| Parameter | Specification |
| --- | --- |
| Package types accommodated | __________________________ |
| Maximum package size | __________________________ |
| Minimum package size | __________________________ |
| X / Y travel | __________________________ |
| Cut depth resolution | __________________________ |
| Positioning repeatability | __________________________ |
| Cutter spindle speed | __________________________ |
| Typical cycle time | __________________________ |

Process capability values above are to be completed from the machine acceptance test record prior to release of this data sheet to a customer or customs authority.

# Shipping

| Parameter | Specification |
| --- | --- |
| Packaging | Export crate |
| Crate dimensions (L × W × H) | __________________________ |
| Gross shipping weight | __________________________ |
| Handling | Two-person lift once uncrated; no forklift required |
| Country of manufacture | United States |

Turnpack, LLC

6922 E Via Northgate, Suite 2, Mesa, AZ 85212  |  +1 (480) 331-1878  |  info@turnpack.com  |  www.turnpack.com

Specifications subject to change. Contact Turnpack to confirm configuration for a specific sales order.