MIDAS D5L6

De-Lidder System

Site Preparation & Installation Checklist

Please complete and return this form prior to scheduled installation.

| Customer Information |  |
| --- | --- |
| Company Name: | Spectrum Control de Mexico |
| Contact Name: |  |
| Phone / Email: |  |
| Installation Address: |  |
| Scheduled Install Date: |  |
| Turnpack Sales Order #: | SO2524915 |
| Customer PO #: | PO.MX32667 |

# 1.  Facility Requirements

| ✓ | Requirement | Specification |
| --- | --- | --- |
| ☐ | Machine footprint (W × D × H) | 24″ × 22″ × 14″ |
| ☐ | Recommended clear work area (W × D × H) | 30″ × 24″ × 26″ (room to rotate for rear access) |
| ☐ | Bench load capacity | ≥ 250 lbs; solid, level, non-vibrating |
| ☐ | Machine weight | 141 lbs (2-person lift) |
| ☐ | Tool storage | 6″ shelf minimum; lockable drawer recommended |
| ☐ | Operating environment | ≤ 40 °C, ≤ 90% RH, non-explosive |
| ☐ | Location | In or adjacent to a microelectronics clean room |

# 2.  Electrical Requirements

| ✓ | Requirement | Specification |
| --- | --- | --- |
| ☐ | Supply voltage (factory-configured) | 110 V AC, 60 Hz, single phase |
| ☐ | Current draw | ______ A (confirm against machine nameplate) |
| ☐ | Circuit | Separate from sensitive instrumentation circuits (frequent motor cycles) |
| ☐ | Connection | NEMA 5-15 grounded outlet; standard North American cord supplied |
| ☐ | Outlet location | Within reach of rear panel |
| ☐ | Grounding | ESD wrist-strap connector to single chassis ground point / neutral |

Note: This unit is factory-configured for 110 V / 60 Hz service. No transformer or plug adapter is required for standard Mexican facility power (127 V nominal, 60 Hz).

# 3.  Process Gases, Air & Water

None required. The de-lidder is a dry process — no compressed air, nitrogen, or water is needed.

# 4.  Vacuum & Particle Control

| ✓ | Requirement | Specification |
| --- | --- | --- |
| ☐ | Particle-control vacuum | Included — self-contained ULPA-filtered unit |
| ☐ | Connection | 1.5″ ID ESD-conductive hose to machine vacuum outlet |
| ☐ | Cleanroom central vacuum (optional) | May connect via 1.5″ conductive outlet in lieu of supplied unit |
| ☐ | Vent / exhaust | Not required (ULPA filtration) |

# 5.  Safety & ESD

| ✓ | Requirement | Specification |
| --- | --- | --- |
| ☐ | Enclosed cutter with dual lid interlocks | Standard — cutter stops when lid opens |
| ☐ | Spindle lock for tool changes | Provided in tool kit |
| ☐ | Safety glasses for operators | Recommended |
| ☐ | ESD controls per microelectronics practice | Customer-provided |

# 6.  Pre-Installation Verification

Please confirm the following items are ready before the installation technician arrives:

| ✓ | Item | Ready? |
| --- | --- | --- |
| ☐ | Bench in place — ≥ 250 lb capacity, level and non-vibrating |  |
| ☐ | 110 V / 60 Hz outlet within reach of rear panel, separate from instrumentation circuits |  |
| ☐ | ~30″ × 24″ clear work area with room to rotate the machine for rear access |  |
| ☐ | Space at bench for the particle-control vacuum unit |  |
| ☐ | ESD ground point available at the bench |  |
| ☐ | Clear delivery path (machine ships in an export crate; 2-person lift) |  |
| ☐ | Personnel available for installation and operator training |  |
| ☐ | Sample devices available for process validation (recommended) |  |

# 7.  Additional Notes / Special Requirements

Additional Notes / Special Requirements

# 8.  Customer Acknowledgment

By signing below, the customer confirms that all site preparation requirements listed above will be completed prior to the scheduled installation date.

| Customer Signature | Date |
| --- | --- |
| Printed Name / Title | Phone / Email |

Questions? Contact your Turnpack representative for assistance with site preparation or installation scheduling.